Removal of Lead Ions by Adsorption onto Seeds’ Peel of Watermelon

A. A. El-Asmy1, S. E. Samra1 and A. M. EL-Nokrashy2

1Chemistry Department, Faculty of Science, Mansoura University, P.O. Box 66,  Mansoura, Egypt.

2 Central Laboratory for drinking water , Mansoura City, Egypt. (a_m_el_1982@hotmail.com).

The Removal of Pb (II) ions from aqueous solutions by adsorption onto seeds’ peel of watermelon (SPW) was investigated. The main parameters (i.e., solution pH, initial concentration of metal ions, sorbent dose, shaking time and temperature) influencing the sorption process, were examined. The Freundlich and Langmuir were applied. Thermodynamic parameters viz. the change in Gibbs free energy change (∆Gº), enthalpy (∆Hº) and entropy (∆Sº) were also calculated. Under the optimum experimental conditions employed, the removal of ~100% of Pb (II) was attained. The procedure was successfully applied to remove Pb (II) from aqueous and different natural water samples.

## Keywords: Lead, sorption, low-cost sorbent, water samples.